Pulsed Light Technology

A relatively new method that uses short intense flashes of ultraviolet-rich, broad-spectrum light has applications ranging from medical manufacturing, to processing water, air, or foods.  The method is effective against all microorganisms (bacteria, fungi, spores, viruses, protozoa, and cysts).  Because of the high peak power of the flash and its ability to provide high doses of broad-spectrum light in very short times, the process is more effective than conventional ultraviolet sources. The pulsed light flashes under consideration are produced using pulsed power engineering technologies to convert alternating current line-voltage to a high voltage direct current pulse for energizing a tubular flashlamp. The light generation process is very efficient in converting electrical to light energy.  Approximately 50-75% of the electrical power energizing the lamp is converted to light emission.  Because of power compression associated with the pulsed power components of the system, average electrical requirements for routine operation are modest (<16 kW), however, peak electrical power into the lamp and the power of the emitted flash are high, being greater than about 4 MW.  The resulting flash is greater than 90,000 times brighter than sunlight with about 25% of its output at wavelengths in the UV.  This presentation will describe the technology and equipment employed and discuss some of the important applications under development.  One focus of the discussion will be results toward the development of pulsed light as a terminal sterilization method for pharmaceutical manufacturing and regulatory perspectives on the validation of new sterilization technologies.

Presenter: Dr. Joseph Dunn

Chief Scientist, R&D

Automatic Liquid Packaging, Inc.

2200 Lakeshore Drive

Woodstock, IL 60098

(815) 206-1291

jdunn@alp-bfs.com